# RouteLab Dynamic Routing Protocol Simulator: Comprehensive Technical & Architectural Guide

> **Course Project**: Team 20 — "Distance Vector, Link State & BGP: A Comparative Routing Simulator"

> **Authors**: Aakarsh, Sravan, Sneha

> **Repository**: [https://github.com/nani224/routing-simulator](https://github.com/nani224/routing-simulator)

> **Document Purpose**: Complete technical breakdown of the RouteLab architecture, protocols, packet movement, node mechanics, failure modes, and simulation controls for project presentations, viva examinations, and code defense.

——————————————————————————————————————————————————

# Table of Contents

[Project Overview & Core Problem Solved](#1-project-overview--core-problem-solved)

[Routing Protocols Implemented](#2-routing-protocols-implemented)

- Distance Vector (Bellman-Ford, RIP-like)

- Link State (Dijkstra SPF, OSPF-like)

- BGP-Lite (Path-Vector, AS_PATH loop detection)

- Flooding Baseline

- Route Poisoning Attack & Secure DV Defense

- Summary Comparison Table

[Packet Movement, Latency, and Route Calculation](#3-packet-movement-latency-and-route-calculation)

[Routers and Nodes: Implementation & Virtualization Model](#4-routers-and-nodes-implementation--virtualization-model)

- Real Networking vs. Simulated Mechanics

[Router Information Popup: Field-by-Field Breakdown](#5-router-information-popup-field-by-field-breakdown)

[Crash and Recovery Internals](#6-crash-and-recovery-internals)

[Simulation Controls & Golden Quill Path Tracer](#7-simulation-controls--golden-quill-path-tracer)

8. [End-to-End System Architecture](#8-end-to-end-system-architecture)

9. [Viva / Presentation Quick Reference Cheatsheet](#9-viva--presentation-quick-reference-cheatsheet)

——————————————————————————————————————————————————

# 1. Project Overview & Core Problem Solved

## What is RouteLab?

**RouteLab** is an interactive, real-time, pluggable routing protocol simulator designed to model, visualize, and benchmark how distributed routing algorithms make decisions across computer networks. It provides a visual "Cartographer's Drafting Table" interface paired with an event-driven networking engine.

## What Problem Does It Solve?

**The "Black Box" Learning Curve of Computer Networking**:

In traditional networking education or testing, routing protocols run as background daemons (`routed`, `frr`, `quagga`, or Cisco IOS/Junos). Students and engineers only see final static tables via CLI commands like `show ip route`. They cannot observe:

- The exchange of intermediate routing control messages traversing links in real time.

- The transient formation and resolution of routing loops.

- How count-to-infinity occurs during sudden fiber cuts or power outages.

- How different algorithms scale in control packet overhead versus convergence time.

**Hardcoded Simulators**:

Most educational simulators (e.g., standard Cisco Packet Tracer labs or ad-hoc Python scripts) hardcode distance vector or Dijkstra algorithms into the simulator's core loop. RouteLab provides **zero hardcoding**: the simulation engine is completely algorithm-agnostic. Any routing protocol runs as a sandboxed plug-and-play plugin implementing the `RoutingProtocol` abstract base class.

**Rigorous Comparative Benchmarking**:

RouteLab allows running different algorithms over **identical network topologies** with **identical event schedules** (e.g., node failure at $t = 2.0\text{s}$, cost modification at $t = 4.0\text{s}$) to evaluate:

- **Convergence Time**: Wall-clock / virtual seconds to establish stable, loop-free shortest paths.

- **Control Message Overhead**: Total packets and bytes exchanged on the wire.

- **Route Flaps**: Total table mutation events.

- **Path Stretch Factor**: Ratio of the actual routed path cost to theoretical graph optimality.

——————————————————————————————————————————————————

# 2. Routing Protocols Implemented

RouteLab implements four core routing protocols and two security/adversarial modules located in `backend/routelab/protocols/` and `backend/routelab/attacks/`.

backend/routelab/
├── protocols/
│   ├── dv.py         -> DistanceVectorProtocol (Bellman-Ford / RIP)
│   ├── ls.py         -> LinkStateProtocol (Dijkstra SPF / OSPF)
│   ├── bgp_lite.py   -> BGPLiteProtocol (Path-Vector / BGP-4)
│   └── flooding.py   -> FloodingProtocol (Controlled Flooding Baseline)
└── attacks/
    ├── poisoning.py  -> PoisoningAttackProtocol (Byzantine False-Metric Injection)
    └── mitigation.py -> SecureDistanceVectorProtocol (Cryptographic Origin Validation)

## 2.1 Distance Vector Protocol (`backend/routelab/protocols/dv.py`)

**Underlying Algorithm**: Distributed Bellman-Ford algorithm (analogous to RIP - Routing Information Protocol, RFC 2453).

**Core Logic**:

- Each router knows only its immediate neighbors and their link costs: $c(x, v)$.

- Periodically (every `periodic_interval_s = 2.0s`) and upon network changes (Triggered Updates), each router sends its full distance vector $D_x = [D_x(y) : y \in V]$ to all active neighbors.

- Upon receiving an advertisement from neighbor $v$, router $x$ updates its table using the Bellman-Ford equation:

$$D_x(y) = \min_v \{ c(x, v) + D_v(y) \}$$

**Loop Prevention & Mitigations**:

- **Split Horizon**: A router does not advertise a route back out the interface through which it learned that route (`if route.next_hop == nbr: continue`).

- **Poison Reverse**: Rather than omitting the route, the router advertises an unreachable metric $\infty$ (configured as `16.0`) back to the neighbor it uses as next-hop (`cost = self.infinity`).

- **Triggered Updates**: Transmits delta updates immediately when link state or metric mutates, rather than waiting for the periodic timer.

- **Hold-down Timers**: When a route fails, the router enters a hold-down state during which it ignores higher-cost alternative paths to prevent transient counting-to-infinity.

## 2.2 Link State Protocol (`backend/routelab/protocols/ls.py`)

**Underlying Algorithm**: Reliable Link State Advertisement (LSA) flooding + Dijkstra's Shortest Path First (SPF) algorithm (analogous to OSPF, RFC 2328).

**Core Logic**:

- **Global Topology Map**: Unlike Distance Vector (where routers only share "distances to destinations"), every Link State router floods Link State Advertisements (`LSA`) describing its local links: `(neighbor_id, cost)`.

- **Reliable Flooding & $O(1)$ Duplicate Suppression**: Each LSA contains an advertisement origin (`adv_router`) and a monotonically increasing sequence number (`seq`). When a router receives an LSA:

```python

current_lsa = self.lsdb.get(adv)

if current_lsa is not None and lsa.seq <= current_lsa.seq:

return  # Drop duplicate or stale LSA immediately

```

New LSAs are saved into the Link State Database (`LSDB`) and forwarded out all other active interfaces except the incoming link (Split-Horizon Flooding).

- **Dijkstra SPF Computation**: When the LSDB changes (`self._spf_dirty = True`), the router constructs an adjacency graph from the LSDB and executes Dijkstra's algorithm using a min-heap priority queue (`heapq`):

- Computes the shortest branch tree rooted at `self._node_id`.

- Identifies the first-hop neighbor along the optimal branch for every destination.

## 2.3 BGP-Lite Protocol (`backend/routelab/protocols/bgp_lite.py`)

**Underlying Algorithm**: Path-Vector Routing (analogous to BGP-4, RFC 4271).

**Core Logic**:

- Used for inter-domain routing between Autonomous Systems (`AS`). Each node is assigned an Autonomous System identifier (`as_id` or `as_number`, e.g., AS100, AS200).

- Routes are advertised with an explicit path vector: `AS_PATH = [AS100, AS200, AS300]`.

**AS_PATH Loop Prevention**:

- Loop detection is mathematical and absolute: If a router receives a BGP advertisement whose `AS_PATH` already contains its own Autonomous System ID, the update is immediately discarded:

```python

if self.as_id in as_path:

logger.debug("BGP loop detected: AS %s already in path %s", self.as_id, as_path)

continue

```

**BGP Decision Process (Best Path Selection)**:

When multiple candidate paths exist for destination $d$, BGP-Lite applies standard RFC tie-breaking:

1. Highest Local Preference (`LOCAL_PREF`, configured via `peer_policies`).

2. Shortest `AS_PATH` length (fewer AS hops).

3. Lowest Multi-Exit Discriminator (`MED`).

4. Lowest interior peer link cost.

5. Deterministic tie-break on peer identifier.

**Explicit Route Withdrawals**: When a link drops or reachability is lost, the router issues a `BGP_UPDATE` containing explicit `withdrawals: [dst]`.

## 2.4 Controlled Flooding Baseline (`backend/routelab/protocols/flooding.py`)

**Underlying Algorithm**: Naive packet broadcast with sequence cache and Time-To-Live (`TTL`).

**Core Logic**:

- Whenever a node wants to announce its presence, it broadcasts a packet to all neighbors with `hops = 0` and `ttl = 16`.

- Neighbors cache `(origin, seq)` in `self.seen_messages`. Duplicate packets are dropped.

- Non-duplicates record the reverse path: `discovered_routes[origin] = (hops + 1, src)`.

- If `ttl > 1`, the packet is decremented and re-flooded out all other links.

**Purpose**: Serves as an educational baseline to show the massive packet overhead of naive flooding compared to Link State or Distance Vector.

## 2.5 Security Modules: Poisoning Attack & Secure DV Defense

**Poisoning Attack (`backend/routelab/attacks/poisoning.py`)**:

- Implements a Byzantine compromised router (`PoisoningAttackProtocol`).

- Periodically broadcasts forged distance vectors claiming direct reachability to a target victim router (e.g., `R6`) with a malicious cost of `1.0`:

```python

vector[self.victim_node] = self.poison_cost  # Forges 1.0

```

- Legitimate routers adopt this false route, funneling traffic into a blackhole or man-in-the-middle intercept.

**Secure DV Defense (`backend/routelab/attacks/mitigation.py`)**:

- Implements `SecureDistanceVectorProtocol` with **Cryptographic Origin Validation** and metric sanity checks:

```python

if dst in self.authorized_origins:

allowed = self.authorized_origins[dst]

if src not in allowed and dst != src:

self.rejected_poison_count += 1

continue  # Drops forged advertisement

```

## Protocol Comparison Summary Table

| Feature / Metric | Distance Vector (RIP) | Link State (OSPF) | BGP-Lite (BGP-4) | Flooding Baseline |

| :--- | :--- | :--- | :--- | :--- |

| **Routing Algorithm** | Distributed Bellman-Ford | Dijkstra Shortest Path First (SPF) | Path-Vector Best Path Selection | Hop-by-Hop Broadcast |

| **Information Shared** | Distance vector to all known destinations | Local link states (`neighbor_id, cost`) | Destination prefix + `AS_PATH` vector | Hop count and sequence origin |

| **Knowledge Scope** | Local neighbors only ("routing by rumor") | Full network topology map in LSDB | Autonomous System path traversal | Hop distance to source |

| **Loop Prevention** | Split Horizon, Poison Reverse, Hold-down | Monotonic LSA sequence numbers | `AS_PATH` loop inspection | Monotonic packet cache `(origin, seq)` |

| **Convergence Speed** | Moderate to Slow (iterative relaxation) | Extremely Fast (immediate LSA flood) | Fast (explicit withdrawals) | Fast, but consumes link bandwidth |

| **Message Overhead** | Low to Moderate (periodic vectors) | Low steady-state, bursty during topology change | Very Low (incremental updates only) | Extremely High ($O(E)$ per packet) |

| **Metric Used** | Additive link costs / hop count | Additive link costs (bandwidth-derived) | Policy (`LOCAL_PREF`, AS hops, MED) | Hop count |

——————————————————————————————————————————————————

# 3. Packet Movement, Latency, and Route Calculation

## How Routing Messages are Created

**Creation**:

When an event occurs (timer tick in `on_timer`, or link change in `on_link_change`), the active protocol calls:

`ctx.send(nbr, payload)` or `ctx.broadcast(payload)`.

**Binary Wire Encoding**:

In `RouterNode.send()` (`backend/routelab/core/node.py`), the message is wrapped into a `Packet` dataclass (`backend/routelab/core/packet.py`):

```python

pkt = Packet(

src=self.node_id,

dst=nbr,

proto=self._protocol.name,

seq=self._seq,

ttl=32,

timestamp=self.clock.now(),

payload=payload,

packet_type=PacketType.CONTROL,

)

```

The packet is serialized using high-performance binary **MessagePack** (`msgpack.packb`), caching its exact wire size (`pkt.byte_size = len(encoded)`).

## How Packets Traverse Links (Latency, Jitter, and Bandwidth)

Packets do not reach the destination instantaneously. They pass through the `LinkEmulator` (`backend/routelab/core/link.py`):

**Link Status Verification**:

If `link.is_up == False`, the packet is dropped immediately (`stats.packets_dropped_down += 1`).

**Stochastic Packet Loss**:

If `random.random() < link.loss_rate`, the packet is dropped (`stats.packets_dropped_loss += 1`).

**Delay Calculation**:

The transmission delay models real physical line properties:

$$\text{Total Delay} = \max\left(0.001, \frac{\text{latency\_ms}}{1000} + \text{jitter\_s}\right) + \frac{\text{pkt\_bytes} \times 8}{\text{bandwidth\_bps}}$$

- $\text{latency\_ms}$: Propagation delay across the medium (e.g., 10ms).

- $\text{jitter\_s}$: Uniform random fluctuation drawn from $[-\text{jitter\_ms}, +\text{jitter\_ms}]$.

- $\frac{\text{pkt\_bytes} \times 8}{\text{bandwidth\_bps}}$: Serialization delay on band-limited links.

**Heap-Based In-Flight Pipeline**:

The packet is wrapped into `_InFlightPacket(delivery_time = now + total_delay, packet=pkt, callback=...)` and inserted into a min-heap priority queue (`heapq.heappush(self._queue, in_flight)`).

A dedicated worker coroutine sleeps until the earliest packet's `delivery_time`, popping it from the heap and invoking the neighbor's `receive_packet(pkt)` callback.

## How Packets are Animated on Screen

The React frontend runs a 60 FPS animation loop driven by `requestAnimationFrame` in `frontend/src/App.tsx`.

The simulator maintains an array of `PacketCourier` objects:

```typescript

interface PacketCourier {

id: string;

src: string;

dst: string;

progress: number; // 0.0 (source) to 1.0 (destination)

speed: number;

protocol: string;

payloadType: 'LSA' | 'VECTOR' | 'BGP';

}

```

On each frame tick, `pkt.progress += pkt.speed * dt * speed_multiplier`.

In `frontend/src/components/TopologyCanvas.tsx`, the 2D Canvas computes the interpolated pixel coordinate:

$$x(t) = x_{\text{src}} + (x_{\text{dst}} - x_{\text{src}}) \times \text{progress}$$

$$y(t) = y_{\text{src}} + (y_{\text{dst}} - y_{\text{src}}) \times \text{progress}$$

The canvas renders a luminous courier particle with an outer pulse ring, color-coded by protocol:

- **Copper / Ochre (`#C87533`)**: Distance Vector

- **Verdigris Green (`#3D8B7A`)**: Link State

- **Cobalt Blue (`#2A5298`)**: BGP-Lite

When `progress >= 1.0`, the courier is removed, the arrival chime plays (Web Audio API synthesis), and the target router's RX counter increments.

## How the Optimal Path is Calculated

**Next-Hop Forwarding**:

Routers do not pre-calculate whole end-to-end paths when forwarding. Instead, each router $u$ inspects its local routing table for destination $d$. If the entry says `next_hop = v`, router $u$ forwards the packet to adjacent neighbor $v$. Router $v$ then performs its own lookup.

**Optimal Path Selection**:

- In **Distance Vector**, shortest paths emerge iteratively through Bellman-Ford equation relaxation.

- In **Link State**, shortest paths are computed directly by running Dijkstra's algorithm over the synchronized LSDB.

- In **BGP-Lite**, path selection follows policy preference (`LOCAL_PREF`) followed by AS-path length.

**Path Tracer Verification**:

The frontend Golden Quill Path Tracer (`frontend/src/components/PathTracerPanel.tsx`) simulates hop-by-hop packet forwarding from a source router to a destination router using the routers' actual computed `next_hop` pointers. If a node points back to an already visited node, the tracer flags **"LOOP DETECTED"**. If a next-hop pointer is missing or points to a crashed router, it flags **"BLACKHOLE"**.

——————————————————————————————————————————————————

# 4. Routers and Nodes: Implementation & Virtualization Model

## Are these actual routers, virtual machines, or software objects?

The nodes labeled **R1, R2, R3...** are **software objects** running inside an asynchronous Python runtime. They are **not** hardware routers, and they are **not** heavyweight operating system Virtual Machines (like VirtualBox, QEMU, or Docker containers).

Each router is an instance of the `RouterNode` class (`backend/routelab/core/node.py`).

+-------------------------------------------------------------------+
|                           RouterNode (R1)                         |
|                                                                   |
|  [Operational State: NodeStatus.RUNNING]                          |
|  [Localhost UDP Socket: 127.0.0.1:52341 (DatagramTransport)]      |
|                                                                   |
|  +-------------------------------------------------------------+  |
|  |           IsolatedProtocolRunner (Security Sandbox)         |  |
|  |  +-------------------------------------------------------+  |  |
|  |  | Protocol Plugin: DistanceVectorProtocol / LinkState   |  |  |
|  |  | Routing Table: { Dest -> DVRoute(cost, next_hop) }    |  |  |
|  |  +-------------------------------------------------------+  |  |
|  +-------------------------------------------------------------+  |
|                                                                   |
|  Links: {"R2": LinkEmulator, "R3": LinkEmulator}                  |
|  Counters: TX=42, RX=38, Flaps=3                                  |
+-------------------------------------------------------------------+

## Real Networking Behavior vs. Simulated Behavior

### What is REAL:

**Real Operating System UDP Sockets**:

During `node.start()`, each router creates an actual operating system datagram socket bound to `127.0.0.1` on an ephemeral OS port using Python's asyncio event loop:

```python

loop = asyncio.get_running_loop()

transport, _ = await loop.create_datagram_endpoint(

lambda: _UDPProtocol(self),

local_addr=(self.bind_host, self.bind_port),  # 127.0.0.1:0

)

self._assigned_port = transport.get_extra_info("sockname")[1]

```

**Real Binary Serialization**:

Messages are not Python object references in memory; they are encoded into raw binary wire bytes via `msgpack` and parsed through `Packet.from_bytes(data)`.

**Real Sandboxing**:

Protocols execute inside an `IsolatedProtocolRunner` (`backend/routelab/plugins/loader.py`). If a protocol crashes (e.g., divide-by-zero or memory fault), the runner intercepts the exception, marks the node as `FAULTY`, and prevents the simulation engine from failing.

**Real Async Concurrency**:

Timers, periodic heartbeats, and packet deliveries execute concurrently using standard Python `asyncio` tasks.

### What is SIMULATED:

**No Physical Cables / Layer-1 PHY**:

Ethernet frames, optical transceivers, and CSMA/CD are not simulated. Packets travel across `LinkEmulator` priority queues.

**Logical Link Topologies**:

Instead of physical switch ports (`eth0`, `ge-0/0/0`), nodes have a dictionary of logical link mappings: `self.links: dict[str, LinkEmulator]`.

**Client-Side Engine (Offline Mode)**:

In standalone browser mode (when the Python backend is disconnected), `frontend/src/App.tsx` and `frontend/src/services/api.ts` execute a high-fidelity client-side simulation engine in TypeScript. It computes shortest-path relaxations at 60 FPS and animates couriers directly on the canvas without needing server round-trips.

——————————————————————————————————————————————————

# 5. Router Information Popup: Field-by-Field Breakdown

When you click on any router (e.g., **R1**), the **Node Inspector Drawer** (`frontend/src/components/NodeInspectorDrawer.tsx`) opens.

+-------------------------------------------------------------+
| (R1) Router R1                        [STATUS: RUNNING] [X] |
+-------------------------------------------------------------+
| [Crash Node]                  TX: 48  RX: 52   Flaps: 3     |
+-------------------------------------------------------------+
| [ Routing Table ]   [ Neighbors (2) ]   [ LSDB View ]       |
+-------------------------------------------------------------+
| DEST   | NEXT HOP | METRIC | PROTOCOL                       |
| R2     | R2       | 1      | [DV]                           |
| R3     | R2       | 3      | [DV]                           |
| R4     | R2       | 4      | [DV]                           |
| R5     | R2       | 6      | [DV]                           |
| R6     | R2       | 6      | [DV]                           |
+-------------------------------------------------------------+

## 1. Header & Telemetry Chips

**STATUS**:

- `STATUS: RUNNING`: Router is operating normally. Protocol timers are running and incoming packets are processed.

- `STATUS: CRASHED`: Router has experienced an abrupt power failure or kernel panic. All active timers are purged, and incoming packets are dropped.

**TX (Packets Transmitted)**:

Cumulative count of all control packets sent by this router across all connected interfaces (`node.packets_sent`).

**RX (Packets Received)**:

Cumulative count of valid packets delivered to this router by link emulators (`node.packets_received`).

**Flaps (Route Flap Count)**:

A **route flap** occurs when a route's metric, next-hop, or reachability state changes. In `RouterNode._check_table_flaps()`:

```python

table_sig = hash(tuple((r.destination, r.next_hop, r.cost) for r in table))

if self._last_table_hash != 0 and table_sig != self._last_table_hash:

self.flaps_count += 1

```

High flaps indicate route instability or oscillations (such as during count-to-infinity).

## 2. Routing Table Tab

**DEST (Destination)**:

The target router ID that this route reaches (e.g., `R2`, `R3`, `R4`, `R5`, `R6`).

**NEXT HOP**:

The immediate neighboring router to which packets addressed to `DEST` should be handed off.

- In our Course 6-Node screenshot example for **R1**, every destination lists `NEXT HOP = R2`. This means R1 reaches the entire network via R2 because link `R1 <-> R2` (cost 1) is cheaper than link `R1 <-> R3` (cost 4).

- If a destination is directly connected and reached through a local interface, it displays `DIRECT`.

**METRIC**:

The cumulative routing cost from this router to `DEST`.

- In the screenshot:

- Path $R1 \to R2$: Metric = $1$

- Path $R1 \to R2 \to R3$: Metric = $1 + 2 = 3$

- Path $R1 \to R2 \to R4$: Metric = $1 + 3 = 4$

- Path $R1 \to R2 \to R3 \to R5$: Metric = $1 + 2 + 3 = 6$

- Path $R1 \to R2 \to R3 \to R5 \to R6$: Metric = $1 + 2 + 3 + 1 = 6$ (or $1 + 3 + 2 = 6$)

- If metric $\ge 16$ (in RIP) or $\ge 9999$, the UI renders in red: `∞ (POISON)`.

**PROTOCOL**:

The protocol module responsible for installing this routing table entry:

- `[DV]`: Distance Vector protocol plugin.

- `[LS]`: Link State protocol plugin.

- `[BG]`: BGP-Lite (displays `AS_PATH`, e.g., `[AS100 AS200]`).

## 3. Neighbors Tab

**Neighbor**: The adjacent node ID connected via a physical/logical link (e.g., `R2`, `R3`).

**COST**: The configured routing metric weight assigned to that link (e.g., `1` for link `R1-R2`, `4` for link `R1-R3`).

**LATENCY**: The simulated transmission delay across that link (e.g., `10ms`).

**STATUS**:

- `ONLINE`: Link is intact; packets can pass.

- `DOWN`: Link is broken (fiber cut or neighbor crash); packets are dropped.

——————————————————————————————————————————————————

# 6. Crash and Recovery Internals

The Node Inspector features **Crash Node** and **Recover Node** buttons.

## What Happens Internally During "Crash Node"?

When `handleCrashNode("R2")` is triggered in the frontend and `inject_node_crash("R2")` executes in the backend engine (`backend/routelab/sim/engine.py`):

1. Node R2 Crashes
                     [status = NodeStatus.CRASHED]
                     [Timers canceled, sockets stopped]
                                  |
                                  v
                  2. Neighbor Notification Loop
           For each neighbor N in [R1, R3, R4]:
           N.notify_link_change("R2", cost=999999, up=False)
                                  |
                                  v
              3. Neighbor Protocol State Adaptation
   - Distance Vector: Invalidate all routes where next_hop == "R2"
                      (Set metric = 16 / Infinity).
                      Trigger immediate Poison Reverse advertisements.
   - Link State:      Originate new LSA with seq + 1 (omitting R2).
                      Flood LSA across surviving links.
                      Re-run Dijkstra SPF to find detour paths.
                                  |
                                  v
              4. Network Convergence Invalidation
           engine._is_converged = False
           Packet couriers in transit to R2 are dropped.

**Abrupt Node Halt**:

`node.crash()` changes `node.status` from `RUNNING` to `CRASHED`. All registered timers (`periodic_update`, `refresh_lsa`) are canceled via `node._cancel_all_timers()`. The node stops emitting packets.

**Neighbor Notification**:

All links connected to the crashed node have their status toggled to `is_up = False`. The engine invokes `notify_link_change(crashed_id, cost=999999, up=False)` on all adjacent peers.

**Routing Table Invalidation**:

- In **Distance Vector**: Surviving neighbors scan their routing tables. Any destination whose `next_hop` was the crashed router is marked with metric $\infty$ (`16.0`). Triggered updates propagate the failure across the network.

- In **Link State**: Neighbors generate a new Link State Advertisement (`LSA`) with an incremented sequence number, removing the crashed node from their link lists. The LSA floods throughout the surviving topology, and each router re-runs Dijkstra's algorithm.

**Packet Movement & Blackholes**:

Any packet couriers currently in transit toward the crashed router reach a dead link and are discarded (`packets_dropped_down`).

**Convergence Reset**:

`_is_converged` is reset to `False`. The convergence clock restarts to measure how long the network takes to re-route around the failure.

## What Happens Internally During "Recover Node"?

`node.recover()` sets `status = NodeStatus.RUNNING` and re-invokes `protocol.on_start()`.

Connected links are restored to `is_up = True`.

The recovered node re-discovers its neighbors and broadcasts its initial routing advertisements.

Neighbors update their adjacency tables, lower their route metrics back to optimal values, and the network achieves a new converged state.

——————————————————————————————————————————————————

# 7. Simulation Controls & Golden Quill Path Tracer

+------------------------------------------------------------------------------------------------------+
| [RouteLab] [Course 6-Node v] [Distance Vector v] [|| Pause] [>> Step] [Ccw Reset] 1.0x [---O---]     |
| [ * CONVERGED (0.620s) ]                           [Compare] [Scenarios] [Attacks] [Logs] [Export]   |
+------------------------------------------------------------------------------------------------------+

## 1. Playback Instruments

**Run / Pause (`handleTogglePlay`)**:

Toggles the simulation clock. In Pause mode, `simTime` stops advancing, packet couriers freeze in mid-flight on the canvas, and protocol timers pause.

**Step Forward (`handleStep`)**:

Advances simulation time by an incremental delta (e.g., $+0.1\text{s}$). Allows step-by-step observation of routing exchanges.

**Reset (`handleReset`)**:

Restores `simTime = 0.0s`, clears active packet couriers, flushes event logs, resets telemetry counters (`tx = 0, rx = 0`), and recalculates baseline routing tables.

**Speed Slider (`0.2x` to `4.0x`)**:

Controls the time multiplier. Modifies both the simulation clock rate and the canvas animation velocity:

```typescript

const simDelta = dt * speed;

setSimTime((t) => t + simDelta);

```

## 2. The CONVERGED Indicator

In the top instrument bar, a parchment seal displays:

**`CONVERGED (0.620s)`** in Verdigris Green when stable.

**`ADAPTING (Flapping...)`** in Amber while routing tables are updating.

**Convergence Criteria (`backend/routelab/sim/engine.py:check_convergence()`)**:

A network is strictly defined as converged if and only if **both** of the following conditions are met:

**Empty In-Flight Queues**:

Every `LinkEmulator` queue has zero packets currently in transit:

$$\sum \text{link.in\_flight\_count}() == 0$$

**Quiescence Window Elapsed**:

No router has updated any routing table entry (no flaps) for a duration $\ge \text{quiescence\_window\_s}$ (default $0.5\text{s}$):

$$t_{\text{now}} - t_{\text{last\_mutation}} \ge \text{quiescence\_window\_s}$$

When both criteria pass, the engine freezes the convergence stopwatch, plays an audio chime, and records the total message count and flap count in the `ConvergenceRecord`.

## 3. Timeline Scrubber (`frontend/src/components/TimelineScrubber.tsx`)

Located at the bottom of the screen:

Displays current simulation time: `currentTime / maxTime` (e.g., `4.25s / 10.0s`).

**Event Markers Overlay**: Vertical color-coded tick marks indicate key events:

- **Red markers**: Link failure or attack injection.

- **Green markers**: Network convergence achieved.

- **Brass markers**: Link cost changes.

Users can drag the scrubber slider or click "Step Back / Step Forward" ($0.5\text{s}$) to inspect historical network states.

## 4. Golden Quill Path Tracer (`frontend/src/components/PathTracerPanel.tsx`)

The Path Tracer tests end-to-end routing reachability across the network:

**Interactive Source & Destination Selectors**:

Allows selecting any source (e.g., `R1`) and destination (e.g., `R6`).

**Hop-by-Hop Trace Algorithm (`computePath`)**:

```typescript

const computePath = (src: string, dst: string) => {

const visited = new Set<string>();

const path = [src];

let curr = src;

while (curr !== dst) {

visited.add(curr);

const entry = allRoutes[curr]?.find((r) => r.destination === dst);

if (!entry || !entry.next_hop) return { path, isBlackhole: true };

const next = entry.next_hop;

if (visited.has(next)) {

path.push(next);

return { path, hasLoop: true }; // Loop detected!

}

path.push(next);

curr = next;

}

return { path, hasLoop: false, isBlackhole: false };

};

```

**Visual Highlighting on Canvas**:

- **Normal Optimal Path**: Traversed links illuminate in glowing golden amber with a subtle canvas blur: `R1 -> R2 -> R3 -> R5 -> R6`.

- **Loop Detected**: If a loop forms (e.g., during count-to-infinity: `R1 -> R2 -> R1`), the path turns crimson red with a flashing warning halo on the looping routers.

- **Blackhole**: If a router drops traffic because it lacks a valid next-hop, an amber blackhole alert appears.

——————————————————————————————————————————————————

# 8. Implementation Architecture

RouteLab connects its frontend and backend across a decoupled, asynchronous architecture:

+--------------------------------------------------------------------------------+
|                         BROWSER CLIENT (React 19 + TypeScript)                 |
|                                                                                |
|  +--------------------+  +----------------------+  +------------------------+  |
|  |   TopologyCanvas   |  | NodeInspectorDrawer  |  |  PathTracerPanel       |  |
|  | (HTML5 Canvas 2D)  |  | (Routes/Flaps/LSDB)  |  |  (Hop-by-Hop Tracer)   |  |
|  +--------------------+  +----------------------+  +------------------------+  |
|           ^                         ^                          ^               |
|           |                         |                          |               |
|           +-------------------------+--------------------------+               |
|                                     |                                          |
|                          App.tsx Main Tick Loop (60 FPS)                       |
|                 [Embedded Client Engine / Fallback Topologies]                 |
+--------------------------------------------------------------------------------+
             |                                              ^
             | REST API Requests                            | WebSocket Deltas
             | (GET/POST /api/v1/...)                       | (/ws/simulation)
             v                                              |
+--------------------------------------------------------------------------------+
|                           PYTHON BACKEND (FastAPI + Asyncio)                   |
|                                                                                |
|  FastAPI Application (server.py)                                               |
|  ├── routes_simulation.py (Start, Pause, Step, Compare)                        |
|  ├── routes_topologies.py (Generate, Import, Export GraphML)                   |
|  ├── routes_plugins.py    (Protocol Registration & Schema Validation)          |
|  └── routes_ws.py         (WebSocket Event Broadcaster & Command Ingestion)    |
|                                                                                |
|  SimulationEngine (backend/routelab/sim/engine.py)                             |
|  ├── Dual Clock: RealtimeClock (Monotonic) / VirtualClock (Deterministic Heap) |
|  ├── Append-Only EventStore (High-Resolution Audit Log)                        |
|  ├── Convergence Detector (In-Flight Queue Counter + Quiescence Window)       |
|  └── LinkEmulator Pipeline (Latency, Jitter, Loss, Bandwidth Heap)             |
|                                                                                |
|  RouterNode Pool (backend/routelab/core/node.py)                               |
|  ├── Ephemeral UDP Sockets (127.0.0.1:port via asyncio.DatagramTransport)      |
|  ├── IsolatedProtocolRunner (Plugin Sandboxing & Fault Interception)           |
|  └── Active Routing Protocols:                                                 |
|      ├── DistanceVectorProtocol (Bellman-Ford, Split Horizon, Poison Reverse)  |
|      ├── LinkStateProtocol      (LSA Flooding, LSDB, Dijkstra SPF)             |
|      ├── BGPLiteProtocol        (Path Vector, AS_PATH Rejection, Local Pref)   |
|      ├── FloodingProtocol       (Sequence Cache, TTL Decrement)                |
|      └── Security Modules       (Poisoning Attack & Secure DV Defense)         |
+--------------------------------------------------------------------------------+

## Component Roles & Communication Flow

**Frontend (`frontend/src/`)**:

- Built with React 19, TypeScript, and Vite.

- Canvas visualization does not rely on third-party heavy charting libraries; it uses a high-performance custom HTML5 Canvas 2D context capable of rendering 100+ nodes and packet couriers at 60 FPS.

- Dual-mode operation:

- **Connected Mode**: Streams state deltas from FastAPI via WebSocket (`/ws/simulation`).

- **Offline / Standalone Mode**: Uses internal graph algorithms in `App.tsx` and `api.ts` to run full interactive simulations directly in the browser.

**REST API (`backend/routelab/api/routes_*.py`)**:

- `/api/v1/simulation/start`: Boots a new simulation session with a given topology and protocol.

- `/api/v1/simulation/step`: Advances the simulation clock deterministically.

- `/api/v1/simulation/compare`: Executes automated head-to-head benchmarks comparing Distance Vector and Link State on identical topologies.

- `/api/v1/topologies`: Lists, creates, and exports topologies in JSON and GraphML formats.

**WebSockets (`backend/routelab/api/routes_ws.py`)**:

- Full-duplex connection at `/ws/simulation`.

- Backend pushes state deltas (link cuts, node flaps, packet hops, convergence events) to the UI at ~50 FPS.

- Client sends commands (`step`, `inject_event`, `ping`) over the socket.

**Engine & Protocols (`backend/routelab/sim/`, `backend/routelab/protocols/`)**:

- Orchestrates time, link delivery queues, and node lifecycle.

- Protocols are strictly decoupled from the engine; they communicate exclusively through the `ProtocolContext` interface (`ctx.send`, `ctx.broadcast`, `ctx.set_timer`).

——————————————————————————————————————————————————

# 9. Viva / Presentation Quick Reference Cheatsheet

Use these concise summary points when answering questions during a viva or presentation:

**What is the primary difference between Distance Vector and Link State in RouteLab?**

* *Answer*: Distance Vector routers share **calculated distances to all destinations** only with immediate neighbors ("routing by rumor"). Link State routers share **local link connectivity states** with the entire network via LSA flooding, building an identical topology map (LSDB) on every router, and then running Dijkstra's algorithm independently.

**How does RouteLab handle Count-to-Infinity?**

* *Answer*: In Distance Vector, if a link breaks and split-horizon is disabled, two neighbors can iteratively bounce incrementing metrics between each other. RouteLab demonstrates this in Course Scenario 3, where metrics count up to 16 ($\infty$). RouteLab solves this using **Split Horizon** (not advertising learned routes back to the source) and **Poison Reverse** (advertising metric 16 back to the next-hop).

**Are the routers actual VMs or containers?**

* *Answer*: No. They are asynchronous Python `RouterNode` software objects running inside Python's asyncio runtime. However, they use **real OS UDP sockets** on `127.0.0.1` and serialize binary packets using **MessagePack**, while physical wires are simulated via `LinkEmulator` priority queues.

**How is Convergence defined in RouteLab?**

* *Answer*: Convergence requires two conditions: (1) all `LinkEmulator` in-flight packet queues must be completely empty ($0$ packets in transit), and (2) no routing table changes (flaps) have occurred for at least the `quiescence_window_s` (default $0.5\text{s}$).

**How does BGP prevent routing loops?**

* *Answer*: BGP is a path-vector protocol. Every route advertisement carries an explicit list of Autonomous Systems it has traversed (`AS_PATH`). If a router sees its own Autonomous System ID inside the `AS_PATH` of an incoming advertisement, it immediately discards the packet, preventing inter-domain loops.

**What is a "Route Flap"?**

* *Answer*: A route flap is any mutation in a routing table entry — specifically when a destination's next-hop changes, its metric increases or decreases, or the route is dropped. RouteLab tracks this monotonically using hashes of `(destination, next_hop, cost)`.

**What is Path Stretch?**

* *Answer*: Path stretch is the ratio of the actual routed path cost to the theoretical shortest path cost computed by an oracle (NetworkX). A stretch factor of $1.0$ indicates perfect optimal routing.

——————————————————————————————————————————————————

*End of RouteLab Technical Guide.*